Lab 1

1   Introduction

1.1   Target

- Know how to read datasheet.

- Design simple circuit with 74 familly.

- Optimize boolean expression by using boolean algebra and K-map.

- Know how to use VOM.

1.2    Contents

- Investigate some IC in 74 familly.

- Optimize boolean expression and implement it by using 74s IC.

2   Exercises

Note: Take pictures for every circuits that are implemented and attach its into your report.

Ex 1:   Investigate the function of 2 in 6 IC that was mentioned. Sources of inputs are switch, destinations of outputs are LEDs. Draw your circuit and fill the two truth tables below.

IC 7408: X = AB

| Input 1 | Input 2 | Output |
| --- | --- | --- |
| 1 | 1 | 1 |
| 1 | 0 | 0 |
| 0 | 1 | 0 |
| 0 | 0 | 0 |

IC 7432: X = A + B

| Input 1 | Input 2 | Output |
| --- | --- | --- |
| 1 | 1 | 1 |
| 1 | 0 | 1 |
| 0 | 1 | 1 |
| 0 | 0 | 0 |

Ex 2:   Optimize these expressions below by using bolean algebra for x, K-map for z. Present each step which you do in the report. Draw the circuit and implement your design.

- x=ABCD+ABCD+ABC+D+AB+CD+AB+C+D + BC+ABC

=ABCD+AB+C+D+ABCD+AB CD+ABCD+BC+ABC

=ABCD+AB+C+D+ABCD+ABCD+AB CD+ABC+ABC

=ABCD+AB+C+D+ABC+AB CD+AB

=ABCD+AB+B+C+D+BC+B CD

=ABCD+A1+C+D+BC+B CD

= ABCD+A.1

=A+BCD

- z=M+NM+PN+P

=MM+MP+NM+NPN+P

=MM N+MPN+MNN+NNP+MM P+MPP+MNP+NPP

=0+ MNP+0+0+0+0+MNP+0

=MNP+MNP

K-Map:

|  | P | P |
| --- | --- | --- |
| M N | 0 | 0 |
| MN | 0 | 1 |
| MN | 1 | 0 |
| MN | 0 | 0 |

Ex 3:   Use 7404s, 7408s, 7432s to implement these boolean expressions below. Draw your circuit and fill the truth table below.

- x=AB+C

- y=A+BC

| A | B | C | x | y | z |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 |  |
| 1 | 1 | 0 | 1 | 1 |  |
| 1 | 0 | 1 | 0 | 0 |  |
| 1 | 0 | 0 | 1 | 1 |  |
| 0 | 1 | 1 | 0 | 0 |  |
| 0 | 1 | 0 | 1 | 1 |  |
| 0 | 0 | 1 | 0 | 0 |  |
| 0 | 0 | 0 | 1 | 0 |  |