## Laboratory 2: Oscilloscope

## Môn: Hệ thống số (TN)

## Lớp: L01

## Nhóm: 7

## GVHD: Kiều Đỗ Nguyên Bình

## Thành viên:	Lê Đức Huy 	1810166

## Nguyễn Gia Huy 	1810173

## Huỳnh Thiên Trình	1810615

## Lê Bá Thông 	1810555

## Lê Duy Bình	51204735

# III. Exercises

Exercice 1: Measurement Propagation Delay

Figure 1: Propagation Delay

Use oscilloscope for measure propagation delay of these following ICs and fill these gaps below:

7400: 4.4 ns

7404: 4.6 ns

7408: 8.8 ns

7432: 8.2 ns

Exercice 2: Egde Detector Circuit

Implement 1 on 3 circuits below and test it by using oscilloscope:

- Use IC 7404 and 7408 to implement edge detector circuit.

- Use IC 7404 and 7432 to implement edge detector circuit.

- Use only IC 7400 to implement edge detector circuit. (+1 bonus)

Figure 2: How edge detector circuit works

Figure 3: Sample edge detector circuit design

Exercice 3: (Extend Exercises) Implement the circuit below using edge detector circuit in Ex2.

Figure 4:  SR-Latch Design

Then, fill the truth table below.

| C | S | R | Q | ¬Q |
| --- | --- | --- | --- | --- |
|  | 0 | 0 | NC | NC |
|  | 0 | 1 | 0 | 1 |
|  | 1 | 0 | 1 | 0 |
|  | 1 | 1 | Invalid | Invalid |
| X | 0 | 0 | X | X |
| X | 0 | 1 | X | X |
| X | 1 | 0 | X | X |
| X | 1 | 1 | X | X |

Table 1:  SR-Latch Truth Table

Test circuit using Oscilloscope (Channel 1 for C signal, Channel 2 for Q signal) and describe the function of this circuit.

The function of SR flip-flop is storing one of the two stable voltage levels (binary value 0 and 1) until a new voltage level is written to. The circuit only changes state when clock pulse is present. This circuit is triggered by positive-going transition of clock signal and has active-HIGH inputs :

- Case 1: When S=R=0 and the clock pulse is applied, the output remains in the present state (no change).

- Case 2: When S=0, R=1 (reset) and the clock pulse is applied, the output at Q is reset to 0 and ¬Q=1.

- Case 3: When S=1 (set), R=0 and the clock pulse is applied, the output at Q is set to 1 and ¬Q=0.

- Case 4: When S=R=1 and the clock pulse is applied, the output state is unpredictable since it will try to set and reset the latch at the same time. This condition should be avoided.

- Case 5: When no clock pulse is applied, the circuit will not respond to any conditions of the inputs, therefore the output remains in the present state.